PERMIT TO FLY: THE NEW RULES

- Commission Regulation (EC) No 1702/2003

- Article 2

Products, parts and appliances certification

- 1. Products, parts and appliances shall be issued certificates as specified in Part 21.

2. By way of derogation from paragraph 1, aircraft, including any installed product, part and appliance, which are not registered in a Member State shall be exempted from the provisions of Subparts H and I of Part 21 and shall also be exempted from Subpart P of Part 21 except when aircraft identification marks are prescribed by a Member State.

Article 2e

Permit to fly

The conditions determined before 28 March 2007 by the Member States for permits to fly or other airworthiness certificate issued for aircraft which did not hold a certificate of airworthiness or restricted certificate of airworthiness issued under this Regulation, are deemed to be determined in accordance with this Regulation, unless the Agency determines before 28 March 2008 that such conditions do not provide for a level of safety equivalent to that required by Regulation (EC) No 1592/2002 or this Regulation.

The permit to fly or other airworthiness certificate issued by Member States before 28 March 2007 for aircraft which did not hold a certificate of airworthiness or restricted certificate of airworthiness issued under this Regulation, are deemed to be a permit to fly issued in accordance with this Regulation until 28 March 2008.

- Commission Regulation (EC) No 2042/2003

- Article 3

Continuing airworthiness requirements

1. The continuing airworthiness of aircraft and components shall be ensured in accordance with the provisions of Annex I.

3. By derogation from paragraph 1, the continuing airworthiness of aircraft holding a permit to fly shall be ensured on the basis of the national regulations of the State of registry specific continuing airworthiness arrangements as defined in the permit to fly issued in accordance with the Annex (Part 21) to Commission Regulation (EC) No 1702/2003.

Part 21	Section A

SUBPART H — CERTIFICATES OF AIRWORTHINESS AND RESTRICTED CERTIFICATES OF AIRWORTHINESS

21A.173	Classification

Airworthiness certificates shall be classified as follows:

(a) ……….

(c) Permits to fly shall be issued to aircraft that do not meet, or have not been shown to meet, applicable certification specifications but are capable of safe flight under defined conditions.

21A.174	 Application

(a) …………

(d) Each application for a permit to fly shall include:

(1) The purpose of the flight(s);

(2) The itineraries or airspace, or both, used for the flight;

(3) Minimum flight crew and its qualification, required to operate the aircraft;

(4) Restrictions for carriage of persons other than flight crew;

(5) The ways in which the aircraft does not comply with the applicable certification specifications;

(6) Any restriction considered necessary for safe operation of the aircraft;

(7) Any other information considered necessary for the purpose of prescribing operating limitations.

21A.179	 Transferability and re-issuance within Member States

(a) ……

(b) Where ownership of an aircraft has changed, and the aircraft has a restricted certificate of airworthiness not conforming to a restricted type-certificate, or a permit to fly, such the airworthiness certificate shall be transferred together with the aircraft provided the aircraft remains on the same register, or issued only with the formal agreement of the competent authority of the Member State of registry to which it is transferred.

21A.185	 Issue of permits to fly

The competent authority of the Member State of registry shall issue a permit to fly after the Agency has found that the aircraft and appropriate associated restrictions compensating for departure from the essential requirements permit the aircraft to perform safely a basic flight. For that purpose, the Agency may make or require the applicant to make appropriate inspections or tests necessary to ensure safety.

- SUBPART P –PERMIT TO FLY

GM to Subpart P

The process allowing a flight under a permit to fly can be described as follows:

1. Flow-chart 1: overview

2. Flow-chart 2: approval of flight conditions

3. Flow-chart 3: issue of permit to fly

4. Flow-chart 4: changes after first issue of permit to fly

Flow-chart 1: overview

Flow-chart 2: approval of flight conditions

Flow-chart 3: issue of permit to fly

Flow-chart 4: changes after first issue of permit to fly

21A.701	Scope

(a)	Permits to fly shall be issued in accordance with this Subpart to aircraft that do not meet, or have not been shown to meet, applicable airworthiness requirements but are capable of safe flight under defined conditions and for the following purposes:

Shall: indicating that when the aircraft meets the conditions of this subpart the PtF shall be issued: not meant to exclude other options

airworthiness requirements: Article 5 of Basic Regulation + implementing rules 1702/2003 and 2042/2003

GM: A certificate of airworthiness or restricted category certificate of airworthiness may not be appropriate for an individual aircraft or aircraft type when it is not practicable to comply with the normal continued airworthiness requirements and the aircraft is to a design standard that is demonstrated to be capable of safe flight under defined conditions. Paragraph 21A.701 identifies cases where the issuance of a (Restricted) Certificate of Airworthiness may not be possible or appropriate and this paragraph provides further information and typical examples for clarification where appropriate: -

(1)	Development;

GM	-	testing of new aircraft or modifications

-	testing of new concepts of airframe, engine propeller and equipment;

-	testing of new operating techniques;

(2)	Showing compliance with regulations or certification specifications;

GM:	certification flight testing for type certification, supplemental type certificates, changes to type certificates or European Technical Standard Order authorisation;

(3)	Design organisations or production organisations crew training;

GM:	Flights for training of crew that will perform design or production flight testing before the design approval and Certificate of Airworthiness (C of A) can be issued.

(4)	Production flight testing of new production aircraft;

GM:	For establishing conformity with the approved design, typically this would be the same program for a number of similar aircraft;

(5)	Flying aircraft under production between production facilities;

GM:	green aircraft ferry for follow on final production.

(6)	Flying the aircraft for customer acceptance;

GM:	Before the aircraft is sold and/or registered.

(7)	Delivering or exporting the aircraft;

GM:	Before the aircraft is registered in the State where the C of A will be issued.

(8)	Flying the aircraft for Authority acceptance;

GM:	In the case of inspection flight test by the authority before the C of A is issued.

(9)	Market survey, including customer’s crew training;

GM:	Flights for the purpose of conducting market survey, sales demonstrations and customer crew training with non type certificated aircraft or aircraft for which conformity has not yet been established or for non-registered a/c and before the Certificate of Airworthiness is issued

(10)	Exhibition and air show;

GM:	Flying the aircraft to an exhibition or show and participating to the exhibition or show before the design approval is issued or before conformity with the approved design has been shown.

(11)	Flying the aircraft to a location where maintenance or airworthiness review are to be performed, or to a place of storage;

GM:	Ferry flights in cases where maintenance is not performed in accordance with approved programmes, where an AD has not been complied with where certain equipment outside the Minimum Equipment List (MEL) is unserviceable or when the aircraft has sustained damage beyond the applicable limits.

(12)	Flying an aircraft at a weight in excess of its maximum certificated takeoff weight for flight beyond the normal range over water, or over land areas where adequate landing facilities or appropriate fuel is not available;

GM:	Oversees ferry flights with additional fuel capacity.

(13)	Record breaking, air racing or similar competition;

GM:	Training flight and positioning flight for this purpose are included

(14)	Flying aircraft meeting the applicable airworthiness requirements before conformity to the environmental requirements has been found;

GM:	Flying an aircraft which has been shown to comply with all applicable airworthiness requirements but not with environmental requirements.

(15)	For non-commercial flying activity on individual non-complex aircraft or types for which a certificate of airworthiness or restricted certificate of airworthiness is not appropriate.

GM:	For aircraft which cannot practically meet all applicable airworthiness requirements, such as certain aircraft without TC-holder (“generically termed orphan aircraft”) or aircraft which have been under national systems of Permit to Fly and have not been shown to meet all applicable requirements. The option of a permit to fly for such an aircraft should only be used if a certificate of airworthiness or restricted certificate of airworthiness cannot be issued due to conditions which our outside the direct control of the aircraft owner, such as the absence of properly certified spare parts.

GM:	Note: The above listing is of cases when a permit to fly MAY be issued; it does not mean that in the described cases a permit to fly MUST be issued. If other legal means are available to allow the intended flight(s) they can also be used.

GM 21A.701
Scope

An aircraft registered outside the Member States and used for flight testing by an organisation which has its principle place of business in a Member State, remains under the authority of its state of registry. The Agency or an appropriately approved design organisation can provide, on request, technical assistance to the state of registry for the issue of a permit to fly, or equivalent authorisation, under the state of registry applicable regulations.

21A.703	Eligibility

Any natural or legal person shall be eligible as an applicant for a permit to fly except for a permit to fly requested for the purpose of 21A.701(a)(15) where the applicant shall be the owner. A person eligible for an application for permit to fly is also eligible for application for the approval of the flight conditions.

GM 21A.703
Applicant for a permit to fly

The applicant for a permit to fly may be a person other than the registered owner of the aircraft. As the holder of this permit will be responsible for ensuring that all the conditions and limitations associated with the permit to fly are continuously satisfied, the applicant for the permit should be a person or organisation suitable for assuming these responsibilities. In particular, the organisations designing, modifying or maintaining the aircraft should normally be the holder of the associated permits to fly.

21A.705	Competent Authority

Notwithstanding 21.1, for the purpose of this Subpart, the ‘Competent Authority’ shall be:

(a) The authority designated by the Member State of registry; or

(b) For unregistered aircraft, the authority designated by the Member State which prescribed the identification marks.

GM 21A.705
Competent Authority

An aircraft registered in a Member State is under the responsibility of this Member State for continuing airworthiness aspects. Consequently, any permit to fly under Part 21 should be issued by that Member State including cases where the aircraft will fly in another State. The permit to fly contains all the conditions and restrictions to ensure safe flight but other airspace and operational rules remain the competence of the authority of the State where the flight will take place. The applicant should therefore also ensure compliance with the relevant regulations of that State.

21A.707	Application for permit to fly

(a) Pursuant to 21A.703 and when the applicant has not been granted the privilege to issue a permit to fly, an application for a permit to fly shall be made to the Competent Authority in a form and manner established by that authority.

(b) Each application for a permit to fly shall include:

(1)	The purpose(s) of the flight(s), in accordance with 21A.701;

(2)	The ways in which the aircraft does not comply with the applicable airworthiness requirements;

(3)	The flight conditions approved in accordance with 21A.710.

(c) Where the flight conditions are not approved at the time of application for a permit to fly, an application for approval of the flight conditions shall be made in accordance with 21A.709.

GM 21A.707(b)
Application

EASA Form 21 (see AMC 21B.520(b)) should be obtained from the Competent Authority.

| Application for Part 21 Permit to Fly |  |  |
| --- | --- | --- |
| 1. Applicant: | [Name of applicant] |  |
| 2. Aircraft nationality and identification marks: |  |  |
| 3. Aircraft owner: |  |  |
| 4. Aircraft manufacturer/type |  | 5. Serial number |
| 6. Purpose of flight [Use terminology of 21A.701(a) and add any additional information for accurate description of the purpose, e.g. place, itinerary, duration…] [For an application due to a change of purpose (ref. 21A.713):reference to initial request and description of new purpose] |  |  |
| 7. Expected target date(s) for the flight(s) and duration |  |  |
| 8. Aircraft configuration as relevant for the permit to fly 8.1 The above aircraft for which a permit to fly is requested is defined in [add reference to the document(s) identifying the configuration of the aircraft. Same as required in AMC 21A.263(c)(6) or AMC 21A.709(b) application form, box 5] 8.2 The aircraft is in the following situation related to its maintenance schedule: [Describe status] |  |  |
| 9. Approval of flight conditions [if not available at the time of application, indicate reference of request for approval] [Reference to: 1. EASA approval, if flight conditions are approved by EASA; or 2. DOA approval form (see AMC 21A.263(c)(6)), if approved under DOA privilege; or 3. Competent Authority approval |  |  |
| 10. Date |  | 11. Name and signature [Authorised signatory] |
| EASA Form 21 |  |  |

21A.708	Flight conditions

Flight conditions include:

(a) The configuration(s) for which the permit to fly is requested;

(b) Any condition or restriction necessary for safe operation of the aircraft, including:

(1)	the conditions or restrictions put on itineraries or airspace, or both, required for the flight(s);

(2)	the conditions and restrictions put on the flight crew to fly the aircraft;

(3)	the restrictions regarding carriage of persons other than flight crew;

(4)	the operating limitations, specific procedures or technical conditions to be met;

(5)	the specific flight test programme (if applicable);

(6)	the specific continuing airworthiness arrangements including maintenance instructions and regime under which they will be performed.

GM 21A.708(b)(6)
Continuing airworthiness

In most cases a simple reference to existing maintenance requirements will suffice for aircraft that have a temporarily invalid C of A.

For other aircraft it will have to be proposed by the applicant as part of the flight conditions. For approved organisations they can be included in their procedures.

(c) The substantiation that the aircraft is capable of safe flight under the conditions or restrictions of subparagraph (b);

GM No. 1 to 21A.708(c)
Safe flight

Safe flight normally means continued safe flight and landing but in some limited cases (e.g. higher risk flight testing) it can mean that the aircraft is able to fly in a manner that will primarily ensure the safety of overflown third parties, the flight crew and, if applicable other occupants.

This definition of “safe flight” should not be interpreted as allowing a test pilot, equipped with a parachute and operating over a sparsely populated area, to set out on a test flight in the full knowledge that there is a high probability of losing the aircraft. The applicant should take reasonable care to minimise safety risks and to be satisfied that there is a reasonable probability that the aircraft will carry out the flight without damage or injury to the aircraft and its occupants or to other property or persons whether in the air or on the ground.

GM No. 2 to 21A.708(c)
Substantiations

The substantiations should include analysis, calculations, tests or other means used to determine under which conditions or restrictions the aircraft can perform safely a flight.

GM No. 3 to 21A.708(c)
Operation of Overweight Aircraft

(d) The method used for the control of the aircraft configuration, in order to remain within the established conditions.

GM 21A.708(d)
Control of aircraft configuration

The applicant should establish a method for the control of any change or repair made to the aircraft, for changes and repairs that do not invalidate the conditions established for the permit to fly.

All other changes should be approved in accordance with 21A.713 and when necessary a new permit to fly should be issued in accordance with 21A.711.

21A.709	Application for approval of flight conditions

(a) Pursuant to 21A.707(c) and when the applicant has not been granted the privilege to approve the flight conditions, an application for approval of the flight conditions shall be made

(1)	when approval of the flight conditions is related to the safety of the design, to the Agency in a form and manner established by the Agency; or

(2)	when approval of the flight conditions is not related to the safety of the design, to the Competent Authority in a form and manner established by that authority.

related to the safety of the design: see AMC 21A.710.

Why “safety”?: to exclude environmental issues.

(b) Each application for approval of the flight conditions shall include:

(1)	the proposed flight conditions; and

(2)	the documentation supporting these conditions ; and

(3)	a declaration that the aircraft is capable of safe flight under the conditions or restrictions of paragraph 21A.708(b).

AMC 21A.709(b)
Submission of documentation supporting the establishment of flight conditions

Together with the application, the documentation required by 21A.709(b) must be submitted with the approval form (EASA Form 18B) defined below, completed with all relevant information. If the complete set of data is not available at the time of application, the missing elements can be provided later. In such cases, the approval form must be provided only when all data are available, to allow the applicant to make the statement required in box 8 of the form.

| FLIGHT CONDITIONS FOR A PERMIT TO FLY – APPROVAL FORM |  |
| --- | --- |
| 1. Applicant [Name of organisation providing the flight conditions and associated substantiations] | 2. Approval form nr. Issue: [number and issue, for traceability purpose] |
| 3. Aircraft manufacturer/type | 4. Serial number(s) |
| 5. Aircraft configuration The above aircraft for which a permit to fly is requested is defined in [add reference to the document(s) identifying the configuration of the aircraft] [For change(s) affecting the initial approval form: description of change(s). This form must be re-issued] |  |
| 6. Substantiations [References to the document(s) justifying that the aircraft (as described in 5.) can perform the intended flight(s) safely under the defined conditions or restrictions.] [For change(s) affecting the initial approval form: reference(s) to additional substantiation(s). This form must be re-issued] |  |
| 7. Conditions/Restrictions The above aircraft must be used with the following conditions or restrictions: [Details of these conditions/restrictions, or reference to relevant document, including specific maintenance instructions and conditions to perform these instructions) |  |
| 8. Statement The flight conditions have been established and justified in accordance with 21A.708. The aircraft has no features and characteristics making it unsafe for the intended operation under the identified conditions and restrictions. |  |
| [when approved under a privilege of an approved organisation] 9. Approved under [ORGANISATION APPROVAL NUMBER]” |  |
| 10. Date of issue | 11. Name and signature [Authorised signatory] |
| [when not approved under a privilege of an approved organisation] 12. Approval and date the appropriate approval: EASA, Competent Authority] |  |
| EASA Form 18B |  |

When the flight conditions are approved under a privilege, this form should be used by the approved organisation to document the approval.

21A.710	Approval of flight conditions

(a) When approval of the flight conditions is related to the safety of the design, the flight conditions shall be approved by:

(1)	the Agency; or

(2)	an appropriately approved design organisation, under the privilege of 21A.263(c)(6).

(b) When approval of the flight conditions is not related to the safety of the design, the flight conditions shall be approved by the Competent Authority, or the appropriately approved organisation that will also issue the permit to fly.

GM 21A.710
Approval of flight conditions

1. The approval of flight conditions is related to the safety of the design, when:

a. the aircraft does not conform to an approved design; or

b. an Airworthiness Limitation, a Certification Maintenance Requirement or an Airworthiness Directive has not been complied with; or

c. the intended flight(s) are outside the approved envelope.

2. Examples when the approval of flight conditions is not related to the safety of the design are:

a. production flight testing for the purpose of conformity establishment;

b. delivery / export flight of a new aircraft the design of which is approved;

c. demonstrating continuing conformity with the standard previously accepted by the Agency for the aircraft or type of aircraft to qualify or re-qualify for a (restricted -) certificate of airworthiness.

(c) Before approving the flight conditions, the Agency, the Competent Authority or the approved organisation must be satisfied that the aircraft is capable of safe flight under the specified conditions and restrictions. The Agency or the Competent Authority may make or require the applicant to make any necessary inspections or tests for that purpose.

21A.711	Issue of a permit to fly

(a) The Competent Authority shall issue a permit to fly:

(1)	upon presentation of the data required by 21A.707; and

(2)	when the conditions of 21A.708 have been approved in accordance with 21A.710; and

(3)	when the Competent Authority, through its own investigations, which may include inspections, or through procedures agreed with the applicant, is satisfied that the aircraft conforms to the design defined under 21A.708 before flight.

(b) An appropriately approved design organisation may issue a permit to fly (EASA Form 20b, see Appendix) under the privilege granted under 21A.263(c)(7),when the conditions of 21A.708 have been approved in accordance with 21A.710.

(c) An appropriately approved production organisation may issue a permit to fly (EASA Form 20b, see Appendix) under the privilege granted under 21A.163(e), when the conditions of 21A.708 have been approved in accordance with 21A.710.

(d) The permit to fly shall specify the purpose(s) and any conditions and restrictions approved under 21A.710. It may include conditions and restrictions prescribed by the Competent Authority outside the scope of the conditions of 21A.708(b);

GM 21A.711(d)
Additional conditions and restrictions

The conditions and restrictions prescribed by the Competent Authority may include airspace restrictions to make the conditions approved under 21A.710 more concrete, or conditions outside the scope of the ones mentioned in 21A.708(b) such as a radio station license.

(e) For permits issued under subparagraph (b) or (c) a copy of the permit to fly shall be submitted to the Competent Authority.

21A.713	Changes

(a) Any change that invalidates the flight conditions or associated substantiation established for the permit to fly shall be approved in accordance with 21A.710. When relevant an application shall be made in accordance with 21A.709.

(b) A change affecting the content of the permit to fly requires the issuance of a new permit to fly in accordance with 21A.711.

GM 21A.713
Changes

Changes to the conditions or associated substantiations that are approved but do not affect the text on the permit to fly do not require issuance of a new permit to fly.

In case a new application is necessary, the substantiation for approval of the flight conditions only needs to address the change.

21A.715	Language

The manuals, placards, listings, and instrument markings and other necessary information required by applicable certification specifications shall be presented in one or more of the official language(s) of the European Community acceptable to the Competent Authority.

21A.719	Transferability

(a) A permit to fly is not transferable.

(b) Notwithstanding subparagraph (a) for a permit to fly issued for the purpose of 21A.701(a)(15), where ownership of an aircraft has changed, the permit to fly shall be transferred together with the aircraft provided the aircraft remains on the same register, or issued only with the agreement of the competent authority of the Member State of registry to which it is transferred.

GM 21A.719
Transfer of a permit to fly

Except for permits to fly issued under 21A.701(a)(15), like aircraft without TC holder, a permit to fly is issued based upon the applicant’s declaration of many aspects of the proposed flight or flights, some of which are specific to the applicant. Accordingly, the basis upon which a permit to fly has been issued necessarily is no longer fully in place when the holder of a permit to fly changes, ownership changes, and/or there is a change of register. Such changes necessitate a new application under 21A.707.

- 21A.721 Inspections

The holder of, or the applicant for, a permit to fly shall provide access to the aircraft concerned at the request of the Competent Authority.

- 21A.723 Duration and continued validity

(a) A permit to fly shall be issued for a maximum of 12 months and shall remain valid subject to:

(1)	compliance with the conditions and restrictions of 21A.711(d) associated to the permit to fly;

(2)	the permit to fly not being surrendered or revoked under 21B.530;

(3)	the aircraft remaining on the same register.

(b) Notwithstanding subparagraph (a) a permit to fly issued for the purpose of 21A.701(a)(15) may be issued for unlimited duration.

(c) Upon surrender or revocation, the permit to fly shall be returned to the Competent Authority.

- 21A.725 Renewal of permit to fly

Renewal of the permit to fly shall be processed as a change in accordance with 21A.713.

- 21A.727 Obligations of the holder of a permit to fly

The holder of a permit to fly shall ensure that all the conditions and restrictions associated with the permit to fly are satisfied and maintained.

- 21A.729 Recordkeeping

(a) All documents produced to establish and justify the flight conditions shall be held by the holder of the approval of the flight conditions at the disposal of the Agency and Competent Authority and shall be retained in order to provide the information necessary to ensure the continued airworthiness of the aircraft.

(b) All documents associated to the issue of permits to fly under the privilege of approved organisations, including inspection records, documents supporting the approval of flight conditions and the permit to fly itself, shall be held by the related approved organisation at the disposal of the Agency or the Competent Authority and shall be retained in order to provide the information necessary to ensure the continued airworthiness of the aircraft.

Part 21	Section B

SUBPART A – GENERAL PROVISIONS

21B.20 Obligations of the competent authority

Each competent authority of the Member State is responsible for the implementation of Section A, Subparts F, G, H, and I and P only for applicants, or holders, whose principal place of business is in its territory.

21B.25	Requirements for the organisation of the competent authority

(a) General:

The Member State shall designate a competent authority with allocated responsibilities for the implementation of Section A, Subparts F, G, H, and I and P with documented procedures, organisation structure and staff.

SUBPART H — CERTIFICATES OF AIRWORTHINESS AND RESTRICTED CERTIFICATES OF AIRWORTHINESS

21B.325	 Issue of airworthiness certificates

(a) The competent authority of the Member State of registry shall, as applicable, issue, or amend a Certificate of Airworthiness (EASA Form 25, see Appendix) or Restricted Certificate of Airworthiness (EASA Form 24, see Appendix) or Permit to Fly (EASA Form 20, see Appendix) without undue delay when it is satisfied that the applicable requirements of Section A, Subpart H are met.

(b) ……….

21B.330 Suspension and revocation of certificates of airworthiness and restricted certificates of airworthiness

(a) ……...

(b) Upon issuance of the notice of suspension and revocation of a certificate of airworthiness or restricted certificate of airworthiness or permit to fly the competent authority of the Member State of registry shall state the reasons for the suspension or revocation and inform the holder of the certificate on its right to appeal.

- SUBPART P —PERMIT TO FLY

- 21B.520 Investigation

(a) The Competent Authority shall perform sufficient investigation activities to justify the issuance, or revocation of the permit to fly.

(b) The Competent Authority shall prepare evaluation procedures covering at least the following elements:

(1)	evaluation of the eligibility of the applicant;

(2)	evaluation of the eligibility of the application;

(3)	evaluation of the documentation received with the application;

(4)	inspection of the aircraft;

(5)	approval of the flight conditions in accordance with 21A.710(b).

AMC 21B.520(b)
Application for a permit to fly

The Competent Authority must receive an application for permit to fly on an EASA Form 21 (see below) completed by the applicant.

21B.525 Issue of permits to fly

The Competent Authority shall issue a permit to fly (EASA Form 20a, see Appendix) when it is satisfied that the applicable requirements of Section A, Subpart P are met.

- 21B.530 Revocation of permits to fly

(a) Upon evidence that any of the conditions specified in 21A.723(a) are not met for a permit to fly it has issued, the Competent Authority shall revoke that permit to fly.

(b) Upon issuance of the notice of revocation of a permit to fly the Competent Authority shall state the reasons for the revocation and inform the holder of the permit to fly on the right to appeal.

- 21B.545 Record keeping

(a) The Competent Authority shall operate a system of record keeping that provides adequate traceability of the process for the issue and revocation of each individual permit to fly.

(b) The records shall at least contain:

(1)	the documents provided by the applicant,

(2)	documents established during the investigation, in which the activities and the final results of the elements defined in 21B.520(b) are stated, and

(3)	a copy of the permit to fly.

(c) The records shall be kept for a minimum of six years after the permit ceases to be valid.

Appendices-–EASA Forms

When the Forms of this Annex are issued in a language other than English they shall include an English translation.

The EASA (« European Aviation Safety Agency ») Forms referred to in the appendices to this Part shall have the following obligatory features. Member States shall ensure that the EASA Forms they issue are recognisable and shall be responsible for having those Forms printed.

Appendix I - EASA Form 1 Authorised release Certificate

Appendix II - EASA Form 15a Airworthiness Review Certificate

Appendix III - EASA Form 20a Permit to Fly

Appendix IV - EASA Form 20b Permit to Fly (issued by approved organisations)

- Appendix V - EASA Form 24 Restricted Certificate of Airworthiness

Appendix VI - EASA Form 25 Certificate of Airworthiness

Appendix VII - EASA Form 45 Noise Certificate

Appendix VIII - EASA Form 52 Aircraft Statement of Conformity

- Appendix IX - EASA Form 53 Certificate of Release to Service

Appendix X - EASA Form 55 Production Organisation Approval Certificate

- Appendix XI - EASA Form 65 Letter of Agreement [Production without POA]”

| Competent authority LOGO | PERMIT TO FLY |  |
| --- | --- | --- |
| (*) |  |  |
| This permit to fly is issued pursuant to Regulation (EC) 1592/2002, Article 5(3)(a) and certifies that the aircraft is capable of safe flight for the purpose and within the conditions listed below and is valid in all Member States This permit is also valid for flight to and within non Member States provided separate approval is obtained from the competent authorities of such States. |  | 1. Nationality and registration marks. |
| 2. Aircraft manufacturer/type |  | 3. Serial number |
| 4. The permit covers [purpose in accordance with 21A.701(a)] |  |  |
| 5. Holder: [in case of a permit to fly issued for the purpose of 21A.701(a)(15) this should state: “the registered owner”] |  |  |
| 6. Conditions/Remarks |  |  |
| 7. Validity period: |  |  |
| 8. Place and date of issue |  | 9. Signature of the competent authority representative |

EASA Form 20a

(*) For use by State of Registry

| Member State of the Competent Authority having issued the organisation approval under which the permit to fly is issued; or ‘EASA’ when approval issued by EASA | PERMIT TO FLY |  |
| --- | --- | --- |
| Name and Address of the organisation issuing the permit to fly |  | (*) |
| This permit to fly is issued pursuant to Regulation (EC) 1592/2002, Article 5(3)(a) and certifies that the aircraft is capable of safe flight for the purpose and within the conditions listed below and is valid in all Member States. This permit is also valid for flight to and within non Member States provided separate approval is obtained from the competent authorities of such States. |  | 1. Nationality and registration marks. |
| 2. Aircraft manufacturer/type |  | 3. Serial number |
| 4. The permit covers [purpose in accordance with 21A.701(a)] |  |  |
| 5. Holder: Organisation issuing the permit to fly |  |  |
| 6. Conditions/Remarks |  |  |
| 7. Validity period: |  |  |
| 8. Place and date of issue |  | 9. Authorised Signature Name Approval Reference No. |

EASA Form 20b

(*) For use by Organisation Approval holder

| Competent authority of a Member State of the European Union or EASA |  | Terms of Approval |  |  | TA: NAA.21G.XXXX |
| --- | --- | --- | --- | --- | --- |
| This document is part of Production Organisation Approval Number NAA.21G.XXXX issued to Company name |  |  |  |  |  |
| Section 1. SCOPE OF WORK: |  |  |  |  |  |
| PRODUCTION OF |  |  | PRODUCTS/CATEGORIES |  |  |
| For details and limitations refer to the Production Organisation Exposition, Section xxx |  |  |  |  |  |
| Section 2. Locations: |  |  |  |  |  |
| Section 3. Privileges: |  |  |  |  |  |
| The Production Organisation is entitled to exercise, within its Terms of Approval and in accordance with the procedures of its Production Organisation Exposition, the privileges set forth in 21A.163. Subject to the following: |  |  |  |  |  |
| [keep only applicable text] |  |  |  |  |  |
| a) Prior to approval of the design of the product an EASA Form 1 may be issued only for conformity purposes. |  |  |  |  |  |
| b) A Statement of Conformity may not be issued for a non approved aircraft |  |  |  |  |  |
| Maintenance may be performed, until compliance with maintenance regulations is required, in accordance with the Production Organisation Exposition Section xxx |  |  |  |  |  |
| Permits to fly may be issued in accordance with the Production Organisation Exposition Section yyy |  |  |  |  |  |
| Date of original issue: | Date of this issue: |  |  | Signed: |  |
|  |  |  |  | For the competent authority or EASA |  |

EASA Form 55 - POA Certificate of Approval - Sheet